GPON OLT (2 PON) WEB

USER MANUAL

Version V1.0

Release Date 2024-8-27

CONTENTS

Chapter 1  System Description	1

1.1 Overview	1

1.1.1 OLT Introduction	1

1.1.2 OS Requirement	1

1.2 Connection	2

Chapter 2  OLT Information	3

2.1 Login	3

2.2 Device Information	3

Chapter 3  OLT Configuration	5

3.1 VLAN	5

3.1.1 Create VLAN	5

3.1.2 VLAN Port	6

3.1.3 QinQ/Translation	6

3.2 Uplink Port	7

3.2.1 Information	7

3.2.2 Configuration	7

3.3 PON	8

3.3.1 Information	8

3.3.2 Configuration	9

3.3.3 Range	9

3.3.4 Perf-Stats Information	10

3.3.5 Perf-Stats Configuration	10

3.4 MAC	11

3.4.1 MAC Table	11

3.4.2 Configuration	11

3.4.3 MAC Flapping Information	12

3.4.4 MAC Flapping Configuration	13

3.4.5 MAC Flapping Port Configuration	13

3.5 LACP	13

3.5.1 Static LACP	13

3.5.2 Dynamic LACP	14

3.6 QoS	15

3.7 ACL	15

3.7.1 IP Filter	15

3.7.2 MAC Filter	16

3.7.3 IP/MAC Filter	16

3.7.4 Ingress Effect Filter	17

3.7.5 Egress Effect Filter	17

3.8 IPv6 ACL	18

3.8.1 IPv6 Filter	18

3.8.2 IPv6/MAC Filter	19

3.8.3 IPv6 Ingress Effect Filter	19

3.8.4 IPv6 Egress Effect Filter	20

3.9 IGMP	20

3.9.1 Group Member	20

3.9.2 Global	21

3.9.3 Port	21

3.9.4 Port User VLAN	22

3.9.5 Port Mrouter	22

3.9.6 Static Group	23

3.10 IPv6 MLD	23

3.10.1 Group Member	23

3.10.2 Global	24

3.10.3 Port	24

3.10.4 Port User VLAN	25

3.10.5 Port Mrouter	25

3.10.6 Static Group	26

3.11 STP	26

3.11.1 Information	27

3.11.2 Global	27

3.11.3 Port	28

3.12 Loopback	28

3.12.1 Information	29

3.12.2 Global	29

3.12.3 Port	30

3.13 DHCP	30

3.13.1 DHCP Server	30

3.13.2 DHCP Relay	32

3.13.3 DHCP Snooping	34

3.14 DHCPv6	39

3.14.1 DHCPv6 Server	39

3.14.2 DHCPv6 Relay	41

3.15 IPv6 SLAAC	42

3.15.1 IPv6 SLAAC	42

3.15.2 IPv6 SLAAC Prefix	43

3.15.3 RDNSS	43

3.16 Route	44

3.16.1 IP	44

3.16.2 Static Route	46

3.16.3 RIP	47

3.16.4 OSPF	51

3.16.5 Key Chain	55

3.16.6 Route Table	56

3.17 IPv6 Route	57

3.17.1 IPv6	57

3.17.2 IPv6 Static Route	58

3.17.3 RIPng	58

3.17.4 OSPF6	62

3.17.5 IPv6 Route Table	65

3.18 PPPoE Intermediate Agent	66

3.18.1 PPPoE Intermediate Agent	66

3.18.2 PPPoE Intermediate Agent Interface	67

3.19 Traffic Policy	67

3.19.1 Policy	67

3.19.2 Classifier	68

3.19.3 Behavior	69

Chapter 4  ONU Configuration	71

4.1 ONU AuthList	71

4.1.1 ONU List	71

4.1.2 ONU Status	81

4.1.3 ONU Optical Info	81

4.1.4 ONU Version Information	81

4.1.5 ONU Manual Add	82

4.1.6 ONU Allowist	82

4.1.7 ONU Statistics	83

4.2 ONU AutoFind	83

4.2.1 Automatic Discovery	83

4.2.2 Aging Time	84

4.3 ONU AutoLearn	84

4.3.1 ONU AutoLearn	84

4.3.2 ONU AutoBind	85

4.3.3 ONU AutoDelete	85

4.3.4 ONU Scheduled Reboot	86

4.4 ONU Upgrade	86

4.4.1 UpLoad Image	86

4.4.2 Manual Upgrade	87

4.4.3 Upgrade Status	87

4.4.4 Auto Upgrade	88

4.4.5 Auto Upgrade Status	88

4.5 Rogue ONU	89

4.6 ONU Common Service	89

Chapter 5  Profile Configuration	93

5.1 ONU Profile	93

5.1.1 Information	93

5.1.2 Add profile	93

5.2 DBA Profile	95

5.2.1 DBA profiles	95

5.2.2 Add profile	96

5.3 Traffic Profile	97

5.3.1 Traffic profiles	97

5.3.2 Add profile	98

5.4 Line Profile	98

5.4.1 Line profile	98

5.4.2 Add profile	99

5.5 Service Profile	103

5.5.1 Service profile	103

5.5.2 Add profile	103

5.6 Alarm Profile	106

5.6.1 Profile Infomation	106

5.6.2 Add Profile	107

5.7 Pri Profile	107

5.7.1 Pri Profile	108

5.7.2 Add Profile	108

5.8 IGMP Profile	109

5.8.1 IGMP Profile	109

5.8.2 Add Profile	110

5.9 Format Profile	111

5.9.1 Format Profile	112

5.9.2 Add Profile	112

5.10 Bind Profile	114

Chapter 6  System Configuration	116

6.1 System Log	116

6.1.1 System Log	116

6.1.2 Alarm	116

6.1.3 Threshold Alarm	117

6.1.4 Syslog Server	118

6.1.5 Syslog Server IPv6	119

6.2 Device Management	120

6.2.1 Firmware Upgrade	120

6.2.2 Device Reboot	121

6.2.3 Config File	122

6.2.4 Advance Config File	122

6.3 User Management	123

6.3.1 User manage	123

6.3.2 Role manage	124

6.4 SNMP	124

6.4.1 SNMP V1/V2	124

6.4.2 SNMP V3	125

6.4.3 SMNP V3 Trap	126

6.4.4 Remote Server	127

6.5 AUX IP	128

6.5.1 AUX IP	128

6.5.2 AUX IPv6	128

6. 6 DNS	129

6.6.1 IPv4 DNS	129

6.6.2 IPv6 DNS	130

6.7 System Time	131

6.7.1 RTC	131

6.7.2 NTP	132

6.8 FAN	133

6.9 Mirror	134

6.10 Login Management	135

6.10.1 Login Access List	135

6.10.2 Service Port	136

6.10.3 Login Timeout	137

6.10.4 Telnet Management	138

6.11 SSH	139

6.11.1 SSH State	139

6.11.2 SSH Enable	140

6.12 Tacacs+	141

6.13 Radius	142

6.14 Dot1x	143

6.14.1 Dot1x Information	143

6.14.2 Dot1x Global Enable	144

6.14.3 Port Configuration	145

6.15 Network Diagnose	146

6.15.1 Ping Diagnose	146

6.15.2 Tracert Diagnose	146

6.15.3 Netstat Diagnose	147

6.15.4 Tcpdump Diagnose	148

6.15.5 Iperf Diagnose	148

Chapter 1  System Description

1.1 Overview

1.1.1 OLT Introduction

The WEB management user manual is for the OLTs listed in Table 1-1.

After you have completed installation, connection and commissioning of the equipment, you can start on configuring various services and functions for the equipment.

Table 1-1 GPON (2 PON) OLT interfaces

| Product |  | 2 ports GPON OLT |
| --- | --- | --- |
| Chassis | Racks | 1U 19 inch standard box |
| 1G/10G Uplink Port | QTY | 4 |
|  | Speed | 2*1G/10G 2*1G auto-negotiation |
|  | Type(Independent) | 2*RJ45 and 2*SFP+ (SFP+ is compatible with 1GE/10GE) |
| PON Port | QTY | 2 |
|  | Physical Interface | 2*GPON |
| Management Ports |  | 1*10/100/1000BASE-T out-band port(AUX), 1*CONSOLE port，1*USB2.0, 1*Type-C USB console. |
| Management Mode |  | SNMP, WEB, and CLI(Console/Telnet/SSH) |

1.1.2 OS Requirement

For OLT management, it supports or requires the following operation system.

Table 1-2 Operation System requirement

| CPU | Memory | DISK | Video Card | Operating System |
| --- | --- | --- | --- | --- |
| Frequency above 2GHz | 2GB Or above | 10GB disk space | 65000 color resolving capability 1024*768 and above | Windows2008 Windows XP Windows 7 Windows 8 Windows 10 Windows 11 |

1.2 Connection

Connect the OLT AUX port to IP network. The OLT default management IP is 192.168.8.200.

Please set your PC IP to 192.168.8.X (e.g.192.168.8.123).

AUX

Chapter 2  OLT Information

2.1 Login

Follow the steps to login:

1. Conform “1.2 Connection” to connect;

2. The device default IP address is 192.168.8.200;

3. Open your web browser, type the device IP in address bar;

4. Entry of the username and password will be prompted. Enter the default login User Name and Password. The default username and password is "admin/Xpon@Olt9417#".

Figure 2.1-1: Login

2.2 Device Information

The OLT ports connection status are shown in the top of the interface, and about the OLT basic information.

OLT InformationDevice Information

This part shows the OLT information such as system name, serial number, hardware version, firmware version, MAC address and system time. The system name can be modified if need.

Figure 2.2-1: Device Information

Chapter 3  OLT Configuration

This section is about the basic service of OLT configuration.

3.1 VLAN

OLT equipment switch engine is fully compliant with the IEEE802.1Q VLAN standard and has the following main features:

- Support Port-based VLAN and IEEE802.1Q VLAN.

- Support full 4K VLAN group, VID range 1~4094.

All switch ports, including uplink ports and downlink ports, support VLAN partition.

VLAN 1 is the system reserved VLAN, it includes all switch ports which are UNTAG mode.

3.1.1 Create VLAN

OLT ConfigurationVLAN

In this user interface, you can create new VLAN.

Figure 3.1-1: Create New VLAN

3.1.2 VLAN Port

OLT ConfigurationVLANVALN Port

Assign the ports to the VLANs that have been created. You can choose the tag or untag VLAN mode.

Figure 3.1-2: Add VLAN Port

3.1.3 QinQ/Translation

OLT ConfigurationVLANQinQ/Translation

In this user interface, VLAN QinQ and VLAN translation can be configured. VLAN QinQ and translation are effective for ingress.

Figure 3.1-3: QinQ/Translation Configuration

3.2 Uplink Port

Uplink ports traffic statistics and basic configuration setting.

3.2.1 Information

OLT ConfigurationUplink PortInformation

This user interface displays traffic statistics of uplink ports.

Figure3.2-1: Uplink Ports Traffic Statistics

3.2.2 Configuration

OLT ConfigurationUplink PortConfiguration

This user interface is used to configure port related functions and characteristic parameters of uplink port, such as port attributes, PVID, speed, rate limit, storm inhibition, port isolation and so on.

Figure3.2-2: Uplink Ports Configuration

Illustrations of each parameter:

| Parameters | Illustration |
| --- | --- |
| Port ID | GE port has two types, fiber SFP (GE1 to GE2) and copper (GE3 to GE4). |
| Description | Descriptions or remarks of port. |
| Admin Status | Active or inactive status of port. It is Enabled by default. |
| Speed | Configuring Port Rate. |
| Isolate | Port isolation with each other. |
| PVID | Default VLAN ID of the port. |
| Broadcast | Broadcast storm inhibition. |
| Multicast | Multicast storm inhibition. |
| Unknown Unicast | Unknown unicast storm inhibition. |
| Ingress Rate | Port ingress rate. |
| Egress Rate | Port egress rate. |

3.3 PON

3.3.1 Information

OLT ConfigurationPONInformation

This user interface is used to displays parameters of PON port, such as PON module port current Temperature, Voltage, Transmit power,Vendor Name,Vendor Revision,Vendor Serial Number.

Figure3.3-1: PON Information

3.3.2 Configuration

OLT ConfigurationPONConfiguration

This user interface is used to configure rate limit, storm inhibition, port isolation and so on like uplink port.

Figure3.3-2: PON configuration

3.3.3 Range

OLT ConfigurationPONRange

When ONU is more than 20km away from OLT, you need to configure PON distance range. The difference between minimum and maximum should not be more than 20km. The unit is 100m.

For example, ONU is 25km away from OLT, the minimum is 50 and the maximum is 250.

Figure3.3-3: PON Range Configuration

3.3.4 Perf-Stats Information

OLT ConfigurationPONPerf-Stats Information

This user interface is used to View the traffic statistics of the PON port within a period of time.

Figure3.3-4: PON Perf-Stats Information

3.3.5 Perf-Stats Configuration

This user interface is used to Set the sampling time and sampling times for traffic statistics.

Figure3.3-5: PON Perf-Stats Configuration

3.4 MAC

In this section, you can check MAC address table of OLT, set MAC aging time and add MAC address manually.

3.4.1 MAC Table

OLT ConfigurationMACMAC Table

This table displays MAC addresses that OLT has learnt at PON ports and GE ports.

Figure3.4-1: MAC Address Table

3.4.2 Configuration

OLT ConfigurationMACConfiguration

The default MAC aging time of OLT is 300s, user can change the value between 10~1000000s. Also, user can add MAC address to the OLT manually.

Figure 3.4-2: MAC Configuration

3.4.3 MAC Flapping Information

This interface displays information learned on multiple ports for the same MAC if you enable MAC Flapping switch.

Figure 3.4-3: MAC Flapping Information

3.4.4 MAC Flapping Configuration

You can enable MAC Flapping Configuration in this interface.

Figure 3.4-4: MAC Flapping Configuration

3.4.5 MAC Flapping Port Configuration

This user interface is used to enable MAC Flapping Configuration for specific port.

Figure 3.4-5: MAC Flapping Port Configuration

3.5 LACP

3.5.1 Static LACP

OLT ConfigurationLACPStatic LACP

To assign and configure an uplink physical interface to a channel group, select load balance for LACP function. When a traffic link can't be used suddenly, the traffic link will switch to another link automatically. The group range is from 1 to 4. Each group can add 4 ports maximally. Only GE ports can be added in the channel groups.

Figure 3.5-1: Create Static LACP

3.5.2 Dynamic LACP

OLT ConfigurationLACPDynamic LACP

This page displays dynamic LACP information. Only the port which is linkup can be shown in the table. OLT can detect how many devices the uplink ports connected to. If the ports are connected to the same device, they will be in a channel group, otherwise in different channel group.

Figure 3.5-2: Dynamic LACP Information

3.6 QoS

OLT ConfigurationQoS

When bandwidth is not enough or there is congestion in the network, queue scheduling can make sure high priority data traffic passes through the device firstly. Traffic will map to queues according to their priorities and transmit in the queues.

OLT supports eight queues altogether. Queue scheduling mode includes strict priority (SP), weighted round robin (WRR) and hybrid mode (SP-WRR).

Strict priority scheduling guarantees high priority traffic occupy as much as bandwidth. The lower priority traffics pass though only when there is remaining bandwidth.

Figure 3.6-1: QoS Configuration

3.7 ACL

In order to filter data packages, network equipment need to setup a series of rules for identifying what need to be filtered. Only matched with the rules the data packages can be filtered. ACL can achieve this function. Matched conditions of ACL rules can be source address, destination address, Ethernet type, VLAN, protocol port, and so on. These ACL rules also can be used in other situations, such as classification of stream in QoS. An ACL rule may contain one or several sub-rules, which have different matched conditions.

This device supports the following types of ACL.

3.7.1 IP Filter

OLT ConfigurationACLIP Filter

The filter is basic on the IP address, including source IP address and destination IP address.

Figure 3.7-1: IP Filter

3.7.2 MAC Filter

OLT ConfigurationACLMAC Filter

The filter is basic on the MAC address, including source MAC address and destination MAC address.

Figure 3.7-2: MAC Filter

3.7.3 IP/MAC Filter

OLT ConfigurationACLIP/MAC Filter

This filter mix the IP address and MAC address, include source MAC address and destination MAC address, source IP address and destination IP address.

Figure 3.7-3: IP/MAC Filter

3.7.4 Ingress Effect Filter

OLT ConfigurationACLEffect Filter

Bind the access list to the ports then it can take effect. Each access list can be bound several ports.The inbound direction of port traffic is bound here.

Figure 3.7-4: Bind Security Filter(Ingress)

3.7.5 Egress Effect Filter

OLT ConfigurationACLEffect Filter

Bind the access list to the ports then it can take effect. Each access list can be bound several ports.The outbound direction of port traffic is bound here.

Figure 3.7-5: Bind Security Filter(Egress)

3.8 IPv6 ACL

This part is about IPv6 security configuration of OLT. IPv6 ACL can permit or deny data passing or accessing by IPv6 packets.

3.8.1 IPv6 Filter

OLT ConfigurationIPv6 ACL IPv6 Filter

The filter is based on the IPv6 address, including source IPv6 address and destination IPv6 address.

Figure 3.8-1: IPv6 Filter

3.8.2 IPv6/MAC Filter

OLT ConfigurationIPv6 ACLIPv6/MAC Filter

This filter mixes IPv6 address, MAC address and other parameters, including source IPv6 address and destination IPv6 address, source MAC address and destination MAC address, VLAN, Ethernet type, protocol, TCP/UDP port, and so on.

Figure 3.8-2: IPv6/MAC Filter

3.8.3 IPv6 Ingress Effect Filter

OLT Configuration  IPv6 ACL IPv6 Ingress Effect Filter

Bind access list to ports so that the ACL rules can take effect. Each access list can be bound to several ports.The inbound direction of port traffic is bound here.

Figure 3.8-3: Bind IPv6 Security Filter(Ingress)

3.8.4 IPv6 Egress Effect Filter

OLT Configuration  IPv6 ACL IPv6 Egress Effect Filter

Bind access list to ports so that the ACL rules can take effect. Each access list can be bound to several ports.The outbound direction of port traffic is bound here.

Figure 3.8-4: Bind IPv6 Security Filter(Egress)

3.9 IGMP

3.9.1 Group Member

OLT ConfigurationIGMPGroup Member

When there is a multicast group produced, the group will display in this table.

Figure 3.9-1: Group Member

3.9.2 Global

OLT Configuration IGMPGlobal

IGMP basic configuration mainly contains parameters of query packet. When IGMP status is enabled, OLT works at IGMP snooping mode. IGMP snooping is the process of listening to Internet Group Management Protocol (IGMP) network traffic. The feature allows a network switch to "listen in" on the IGMP conversation between hosts and routers. By listening to these conversations, the switch maintains a map of which devices need which IP multicast streams. Multicasts may be filtered from the ports which do not need them and thus controls which ports receive specific multicast traffic. When IGMP status is disabled, OLT works at transparent mode.

Figure 3.9-2: IGMP Global

3.9.3 Port

OLT Configuration IGMPPort

This configuration is used to set the maximum number of multicast groups, filter and fast leave mode.

Figure 3.9-3: IGMP Port

3.9.4 Port User VLAN

OLT Configuration IGMPPort User VLAN

This configuration is used to configure IGMP VLAN for OLT. Generally, PON ports should be configured, and user VLAN and group VLAN are the same. If user VLAN and group VLAN are different, multicast VLAN will be translated.

Figure 3.9-4: IGMP Port User VLAN

3.9.5 Port Mrouter

OLT Configuration IGMPPort Mrouter

Multicast router port is used to transmit IGMP signal messages. Generally, OLT uplink ports should be set as multicast router ports.

Figure 3.9-5: IGMP Port Mroute

3.9.6 Static Group

OLT Configuration IGMPStatic Group

This configuration is used to bind multicast IP address and VLAN ID.

Figure 3.9-6: IGMP Static Group

3.10 IPv6 MLD

3.10.1 Group Member

OLT Configuration  IPv6 MLD  Group Member

This page displays IPv6 multicast group member ports.

Figure 3.10-1: IPv6 MLD Group Member

3.10.2 Global

OLT Configuration  IPv6 MLD  Global

This page is used to enable IPv6 MLD and set IPv6 MLD related parameters.

Figure 3.10-2: IPv6 MLD Global

3.10.3 Port

OLT Configuration  IPv6 MLD  Port

This page is used to configure group limit value, fast leave for each port.

Figure 3.10-3: IPv6 MLD Port

3.10.4 Port User VLAN

OLT ConfigurationIPv6 MLDPort User VLAN

This page is used to configure MLD VLAN for OLT.

Figure 3.10-4: IPv6 Port User VLAN

3.10.5 Port Mrouter

OLT Configuration  IPv6 MLD  Port Mrouter

This page is used to set a port as IPv6 multicast router port.

Figure 3.10-5: IPv6 MLD Port Mrouter

3.10.6 Static Group

OLT Configuration  IPv6 MLD Static Group

This configuration is used to bind multicast IPv6 address and VLAN ID.

Figure 3.10-6: MLD Static Group

3.11 STP

Spanning Tree Protocol is layer2 protocol, which is used to eliminate network loop by blocking network redundant links selectively. It has the feature of link backup as well.

3.11.1 Information

OLT ConfigurationSTPInformation

Global information mainly displays STP parameters of root bridge device.

Figure 3.11-1: STP Information

3.11.2 Global

OLT ConfigurationSTPGlobal

This configuration is used to set STP parameters of the device, which contains STP switch, priority, hello time, max age, forward delay and MAC address.

Figure 3.11-2: STP Global Setup

3.11.3 Port

OLT ConfigurationSTPPort

This user interface is used to set port STP parameters which contain STP switch, priority, cost, edge port and p2p port.

Figure 3.11-3: STP Port Settings

3.12 Loopback

Loopback can detect loop ports and process loop ports.

3.12.1 Information

OLT ConfigurationLoopbackInformation

Figure 3.12-1: Loopback Information

3.12.2 Global

OLT ConfigurationLoopbackGlobal

This page is used to enable or disable loopback detect and configure loopback mode, age time.

Figure 3.12-2: Loopback Global

3.12.3 Port

OLT ConfigurationLoopbackPort

Loopback port configuration is used to specify the port range of loopback function. Loopback will take effect on the port when it is checked.

Figure 3.12-3: Loopback Port

3.13 DHCP

OLT can support the following DHCP functions.

- DHCP Server

- DHCP Relay

- DHCP Snooping

3.13.1 DHCP Server

3.13.1.1 DHCP Bind Information

OLT ConfigurationDHCPDHCP ServerDHCP Bind Information

This table displays the MAC addresses, host name and IP addresses, lease time assigned to them.

Figure 3.13-1: DHCP Bind Information

3.13.1.2 DHCP Server Enable

OLT ConfigurationDHCPDHCP ServerDHCP Server Enable

This parameter is used to configure different DHCP servers for different VLAN ID.

Before enabling DHCP server, you must configure IP address for the VLAN.

Figure 3.13-2: DHCP Server Enable

3.13.1.3 DHCP Pool Configuration

OLT ConfigurationDHCPDHCP ServerDHCP Pool Configuration

This page is used to configure basic information about the DHCP server, including Pool Name, Lease Time, and Gateway.

Figure 3.13-3: DHCP Pool Configuration

3.13.2 DHCP Relay

3.13.2.1 Configuration

OLT ConfigurationDHCPDHCP RelayConfiguration

Because the DHCP service exists in one broadcast domain, the server and the client are usually in the same network segment. DHCP relay can solve the issue that DHCP server and client do not exist in the same network segment.

Figure 3.13-4: DHCP Relay Configuration

3.13.2.2 Global

OLT ConfigurationDHCPDHCP RelayGlobal

The global configuration of DHCP relay mainly includes option 82 settings, including switches and processing modes.

Figure 3.13-5: DHCP Relay Global

3.13.2.3 Port

OLT ConfigurationDHCPDHCP RelayPort

This user interface is used to configure the parameters "Option 82 Circuit ID" and "Option 82 Remote ID" for option 82.

Figure 3.13-6: DHCP Relay Port

3.13.3 DHCP Snooping

3.13.3.1 Bind List

OLT ConfigurationDHCPDHCP SnoopingBind List

The static bind of the DHCP Snooping will be shown in the table.

Figure 3.13-7: DHCP Snooping Bind List

3.13.3.2 Global

OLT ConfigurationDHCPDHCP SnoopingGlobal

DHCP Snooping is used to prevent the DHCP message attacking and guarantee network to get a correct IP address.

DHCP snooping global configuration mainly contains option 82 settings, DHCP traffic rate limit and snooping VLAN.

Figure 3.13-8: DHCP Snooping Global

3.13.3.3 Port

OLT ConfigurationDHCPDHCP SnoopingPort

This user interface is used to configure DHCP snooping parameters of ports which contain port type, option 82 parameters and rate limit.

All the ports are untrust ports by default. Option82 parameters, “Option 82 Circuit ID” and “Option 82 Remote ID”, are effective for untrust ports. “Limit Rate” is the ports’ max speed of receiving DHCP packets.

Figure 3.13-9: DHCP Snooping Port Setup

3.13.3.4 Static Bind

OLT ConfigurationDHCPDHCP SnoopingStatic Bind

DHCP snooping binding is useful when a host needs a fixed IP address assigned by DHCP server from the specific port.

Figure 3.13-10: DHCP Snooping Static Bind

3.13.3.5 IP Source Guard

Only GPON OLT-B Series supports this feature.

OLT ConfigurationDHCPDHCP SnoopingIP Source Guard

This function is actually based on the DHCP Snooping Bind List to restrict access to the external network .That means that an issue outside the list cannot access the external network

Figure 3.13-11: DHCP Snooping IP Source Guard

3.13.3.6 IP Source Bind

Only GPON OLT-B Series supports this feature.

OLT ConfigurationDHCPDHCP SnoopingIP Source Bind

If you configure a rule in IP Source Guard, a dynamic rule is displayed in IP Source Bind Table.You can add a static rule manually on this page.It works as described in the previous section.

Figure 3.13-12: DHCP Snooping IP Source Bind

3.14 DHCPv6

3.14.1 DHCPv6 Server

DHCPv6 is a network protocol that used to configure IPv6 address, IPv6 prefix, DNS, domain and other network parameters for a host which operating on an IPv6 network.

3.14.1.1 DHCPv6 Bind Information

OLT Configuration  DHCPv6  DHCPv6 Server  DHCPv6 Bind Information

DHCPv6 bind information displays IPv6 addresses which have been assigned to hosts.

Figure 3.14-1: DHCPv6 Bind Information

3.14.1.2 DHCPv6 Server Enable

OLT Configuration  DHCPv6  DHCPv6 Server  DHCPv6 Server Enable

Select VLAN and fill in DHCPv6 pool name, enable DHCPv6 server, then the VLAN will be added into the table. Before enabled DHCPv6 server, VLAN IPv6 address and server pool are required.

Figure 3.14-2: DHCPv6 Server

3.14.1.3 Server Pool Configuration

OLT Configuration  DHCPv6  DHCPv6 Server  Server Pool Configuration

DHCPv6 pool specifies the range of assigned IPv6 address. Life time, DNS and domain also can be specified here for DHCPv6 client.

Figure 3.14-3: DHCPv6 Pool

3.14.1.4 DHCPv6 DUID

OLT Configuration  DHCPv6  DHCPv6 Server  DHCPv6 DUID

This page is used to configure DHCPv6 DUID types, Enterprise Number and Identifier.

Figure 3.14-4: DHCPv6 DUID

3.14.2 DHCPv6 Relay

OLT Configuration  DHCPv6  DHCPv6 Relay  Configuration

During the process of obtaining the IPv6 address/prefix and other network configuration parameters dynamically through the DHCPv6 relay, the DHCPv6 client and the DHCPv6 server are processed in the same way as when the DHCPv6 relay is not processed.

Figure 3.14-5: DHCPv6 Relay

3.15 IPv6 SLAAC

IPv6 network uses the ICMPv6 route discovery protocol. When an IPv6 host connects to the network for the first time, it automatically configures it according to the information got by route discovery/prefix discovery. Route discovery/prefix discovery is that when a host is connected to IPv6 network, it can discover local router and obtain neighbor information, prefix of current network and other configuration parameters from route advertisement (RA) packets.

3.15.1 IPv6 SLAAC

OLT Configuration  IPv6 SLAAC  IPv6 SLAAC

When IPv6 host use SLAAC (Stateless Address AutoConfiguration), OLT will send a route advertisement (RA) packet to it. This page is used to configure parameters of the route advertisement packet.

Figure 3.15-1: IPv6 SLAAC

3.15.2 IPv6 SLAAC Prefix

OLT Configuration  IPv6 SLAAC  IPv6 SLAAC Prefix

When IPv6 host uses stateless address auto configuration, OLT can provide IPv6 prefix. The host will generate an IPv6 address with the prefix.

Figure 3.15-2: IPv6 SLAAC Prefix

3.15.3 RDNSS

OLT Configuration  IPv6 SLAAC  RDNSS

OLT will send the route advertisement packet with the DNS parameters you configured.

Figure 3.15-3: RDNSS

3.16 Route

3.16.1 IP

3.16.1.1 VLAN IP

OLT ConfigurationRouteIPVLAN IP

This configuration is used to configure IP address for VLAN. When the VLAN is added to a port, you can access OLT by the IP address from the port.

Figure 3.16-1: VLAN IP

3.16.1.2 ARP Proxy

ARP Proxy is a technique by which a device on a given network answers the ARP queries for a network address that is not on that network. The ARP Proxy is aware of the location of the traffic's destination, and offers its own MAC address as (ostensibly final) destination. The "captured" traffic is then typically routed by the Proxy to the intended destination via another interface or via a tunnel.

The process which results in the node responding with its own MAC address to an ARP request for a different IP address for proxying purposes is sometimes referred to as 'publishing'. Not all OLT models support ARP Proxy.

OLT ConfigurationRouteIPARP Proxy

Figure 3.16-2: ARP proxy configuration

3.16.2 Static Route

Static route is a form of routing that a router uses a manually-configured routing entry. In many cases, static routes are manually configured by a network administrator. Unlike dynamic routing, static routes are fixed and do not change if the network is changed or reconfigured.

The OLT only supports static route. After configured VLAN IP address, add static routes to make the network on the different network segment communicate with each other.

OLT ConfigurationRouteStatic Route

Figure 3.16-3: Static Route

3.16.3 RIP

RIP (Routing Information Protocol) is a simple internal gateway protocol, which is based on the D-V algorithm and uses hop count to represent metric. The hop count is the number of routers that a datagram must pass through. RIP only support maximum 15 hops; hence it is fit for a small network.

3.16.3.1 RIP Information

OLT Configuration  Route  RIP  RIP Information

This page displays RIP information.

Figure 3.16-4: RIP Information

3.16.3.2 RIP Enable

OLT Configuration  Route  RIP  RIP Enable

Enable RIP protocol and configure RIP parameters.

Figure 3.16-5: RIP Enable

3.16.3.3 RIP Route Networking

OLT Configuration  Route  RIP  RIP Route Networking

This page is used to add RIP route networking. VLAN IP address must be set before adding the VLAN to RIP route networking table.

Figure 3.16-6: RIP Route Networking

3.16.3.4 RIP Redistribute

OLT Configuration  Route  RIP  RIP Redistribute.

This page is used to enable or disable route redistribute and choose redistribute mode.

Figure 3.16-7: RIP Redistribute

3.16.3.5 RIP Interface

OLT Configuration  Route  RIP  RIP Interface

This page is used to configure RIP interface and its authentication type. VLAN IP address must be set before configuring RIP interface. And auth chain should be set on page Key Chain, refer to section 3.16.5.

Figure 3.16-8: RIP Interface

3.16.4 OSPF

OSPF (Open Shortest Path First) is an internal gateway protocol based on link state routing protocol. This protocol uses the Dijkstra algorithm to calculate the shortest path to each network, and performs the algorithm to quickly converge to the new loop-free topology when detecting changes in the link (such as link failure).

3.16.4.1 OSPF Information

OLT Configuration  Route  OSPF  OSPF Information

This page displays OSPF information, including neighbor information and OSPF routing information.

Figure3.16-9: OSPF Information

3.16.4.2 OSPF Enable

OLT Configuration  Route  OSPF  OSPF Enable

This page is used to enable OSPF. Fill in route ID and let it blank, enable OSPF. OLT will use the biggest IP address as route ID if it’s blank.

Figure 3.16-10: OSPF Enable

3.16.4.3 OSPF Route Networking

OLT Configuration  Route  OSPF  OSPF Route Networking

This page is used to configure area number for VLAN where OSPF protocol is operating.

Figure 3.16-11: OSPF Route Networking

3.16.4.4 OSPF Area Type

OLT Configuration  Route  OSPF  OSPF Area Type

This page is used to configure area type. Backbone area will not display on this page.

Figure 3.16-12: OSPF Area Type

3.16.4.5 OSPF Area Summary

OLT Configuration  Route  OSPF  OSPF Area Summary

This page is used to configure area IP address summary.

Figure 3.16-13: OSPF Area Summary

3.16.4.6 OSPF Redistribute

The router can use route redistribution to broadcast the OSPF routing it learns through another routing protocol so that several routing protocols can cooperate with each other in a network.

OLT Configuration  Route  OSPF  OSPF Redistribute

Figure 3.16-14: OSPF Redistribute

3.16.4.7 OSPF Interface

OLT Configuration  Route  OSPF  OSPF Interface

This page is used to OSPF interface parameters such as cost, time, priority, authentication, and so on.

Figure 3.16-15: OSPF Interface

3.16.5 Key Chain

Key management is a method of controlling the authentication key used by routing protocols. The authentication key is available for EIGRP and RIP version 2. To manage the authentication key needs a key chain. Each key has its own key identifier, which is stored locally. The combination of the key identifier and the interface associated with the message uniquely identifies the authentication algorithm and MD5 authentication key in use.

OLT Configuration  Route  Key Chain

Figure 3.16-16: Key Chain

3.16.6 Route Table

OLT Configuration  Route  Route Table

This page displays routing items of OLT.

Figure 3.16-17: Route Table

3.17 IPv6 Route

3.17.1 IPv6

OLT Configuration  IPv6 Route  IPv6  VLAN IPv6

Configure IPv6 address for VLAN that has been created.

Figure 3.17-1: VLAN IPv6

3.17.2 IPv6 Static Route

Static route is added manually. It will not change even the situation and network topology has been changed.

OLT Configuration  IPv6 Route  IPv6 Static Route

Add IPv6 static route item one by one.

Figure 3.17-2: IPv6 Static Route

3.17.3 RIPng

RIPng is the next generation of RIP, which uses the IPv6 protocol on top of RIP.

3.17.3.1 RIPng Information

OLT Configuration IPv6 Route  RIPng  RIPng Information

This page displays RIPng information.

Figure 3.17-3: RIPng Information

3.17.3.2 RIPng Enable

OLT Configuration  IPv6 Route  RIPng  RIPng Enable

Enable RIPng protocol and configure RIPng parameters.

Figure 3.17-4: RIPng Enable

3.17.3.3 RIPng Route Networking

OLT Configuration  IPv6 Route  RIPng  RIPng Route Networking

This page is used to add RIPng route networking. VLAN IP address must be set before adding the VLAN to RIPng route networking table.

Figure 3.17-5: RIPng Route Networking

3.17.3.4 RIPng Aggregate

OLT Configuration  IPv6 Route  RIPng  RIPng Aggregate.

This page is used to aggregate routes of the same network segment with different prefixes.

Figure 3.17-6: RIPng Aggregate

3.17.3.5 RIPng Redistribute

OLT Configuration  IPv6 Route  RIPng  RIPng Redistribute.

This page is used to enable or disable route redistribute and choose redistribute mode.

Figure 3.17-7: RIPng Redistribute

3.17.3.6 RIPng Interface

OLT Configuration  IPv6 Route  RIPng  RIPng Interface

This page is used to configure the RIPng interface's modes, including split-horizon, split-horizon, and poisoned-reverse mixed mode selection.

Figure 3.17-8: RIPng Interface

3.17.4 OSPF6

OSPF6 is the next generation of OSPF, which uses the IPv6 protocol on top of OSPF.

3.17.4.1 OSPF6 Information

OLT Configuration  IPv6 Route  OSPF6  OSPF6 Information

This page displays OSPF6 information, including neighbor information and OSPF6 routing information.

Figure3.17-9: OSPF6 Information

3.17.4.2 OSPF6 Enable

OLT Configuration  IPv6 Route  OSPF6  OSPF6 Enable

This page is used to enable OSPF6. Fill in route ID and let it blank, enable OSPF6. OLT will use the biggest IPv6 address as route ID if it’s blank.

Figure 3.17-10: OSPF6 Enable

3.17.4.3 OSPF6 Route Networking

OLT Configuration IPv6 Route  OSPF6  OSPF6 Route Networking

This page is used to configure area number for VLAN where OSPF6 protocol is operating.

Figure 3.17-11: OSPF6 Route Networking

3.17.4.4 OSPF6 Area Summary

OLT Configuration  IPv6 Route  OSPF6  OSPF6 Area Summary

This page is used to configure area IPv6 address summary.

Figure 3.17-12: OSPF6 Area Summary

3.17.4.5 OSPF6 Redistribute

The router can use route redistribution to broadcast the OSPF6 routing it learns through another routing protocol so that several routing protocols can cooperate with each other in a network.

OLT Configuration IPv6 Route  OSPF6  OSPF6 Redistribute

Figure 3.17-13: OSPF6 Redistribute

3.17.4.6 OSPF6 Interface

OLT Configuration  IPv6 Route  OSPF6  OSPF6 Interface

This page is used to OSPF6 interface parameters such as cost, time, priority, authentication, and so on.

Figure 3.17-14: OSPF6 Interface

3.17.5 IPv6 Route Table

OLT Configuration  IPv6 Route  IPv6 Route Table

This table displays all IPv6 route items of the device, including static route and dynamic route.

Figure 3.17-15: IPv6 Route Table

3.18 PPPoE Intermediate Agent

The PPPoE Intermediate Agent intercepts the PPPoE client's PPPoE message and adds port information (such as slot number/sub card number/interface number, VLAN, MAC address, etc.) for the end user's host access to it. It then forwards it to the PPPoE Server through PAD (PPPoE Active Discovery) to achieve binding authentication between the end user's user account and the access port.

3.18.1 PPPoE Intermediate Agent

OLT Configuration  PPPoE Intermediate Agent  PPPoE Intermediate Agent

This page is used to configure the PPPoE Intermediate Agent's switches, bound VLANs, and other parameter information.

Figure 3.18-1: PPPoE Intermediate Agent

3.18.2 PPPoE Intermediate Agent Interface

OLT Configuration  PPPoE Intermediate Agent  PPPoE Intermediate Agent Interface

This page is used to configure whether the port is trusted, as well as the values of Circuit ID and Remote ID.

Figure 3.18-2: PPPoE Intermediate Agent Interface

3.19 Traffic Policy

3.19.1 Policy

OLT Configuration  Traffic Policy  Policy

This page is used to bind flow classifier and flow behavior to flow policy.

Figure 3.19-1: Policy

3.19.2 Classifier

OLT Configuration  TrafficPolicy  Classifier

This page is used to configure flow classifier, It is created based on ACL.

Figure 3.19-2: Classifier

3.19.3 Behavior

OLT Configuration  TrafficPolicy  Behavior

This page is used to configure flow behavior, here subsequent operations can be specified, such as mirroring, speed limits, and data statistics.

Figure 3.19-3: Behavior

Chapter 4  ONU Configuration

This chapter is about the ONU management by OLT.

4.1 ONU AuthList

4.1.1 ONU List

ONU ConfigurationONU AuthListONU List

Select PON port ID, all ONUs will be displayed in this interface. You can check ONU using profile, Registration mode and do some operations to every ONU.

Figure 4.1-1: ONU List

4.1.1.1 Config

ONU ConfigurationONU AuthListONU ListConfig

Configure ONU parameter information which you selected.

Figure 4.1-2: Configure ONU

4.1.1.1.1 Tcont

ONU ConfigurationONU AuthListONU ListConfigTcont

Create tcont ID and bind DBA profile. Tcont name is optional.

Figure 4.1-3: Create Tcont

4.1.1.1.2 Gemport

ONU ConfigurationONU AuthListONU ListConfigGemport

Create gemport ID and bind tcont ID.

Figure 4.1-4: Create gemport

4.1.1.1.3 Service

ONU ConfigurationONU AuthListONU ListConfigService

Create a service, set the VLAN and VLAN mode and bind one gemport ID.

Figure 4.1-5: Create service

4.1.1.1.4 Service Port

ONU ConfigurationONU AuthListONU ListConfigService Port

Create a service port, set the user VLAN and translate VLAN and bind one gemport ID. If don’t need VLAN translation, just set translate VLAN the same as user VLAN.

Figure 4.1-6: Create service port

4.1.1.1.5 PortVlan

ONU ConfigurationONU AuthListONU ListConfig PortVlan

Set the VLAN mode of the ONU’s port. For HGU, need to configure veip 1 transparent; for SFU, configure Ethernet port directly.

Figure 4.1-7: Configure port VLAN mode

4.1.1.1.6 Multicast

ONU ConfigurationONU AuthListONU ListConfig Multicast

Set the Multicast VLAN of ONU and the Multicast VLAN mode of ONU’s port.

Figure 4.1-8: Configure multicast VLAN

4.1.1.1.7 Port

ONU ConfigurationONU AuthListONU ListConfigPort

Set attribute of ONU LAN port.

Figure 4.1-9: ONU port attibute

4.1.1.1.8 Iphost

ONU ConfigurationONU AuthListONU ListConfigIphost

Create Iphost for ONU wan connection. It is used for ONU management.

Figure 4.1-10: Configure IPhost

4.1.1.1.9 WAN

ONU ConfigurationONU AuthListONU ListConfigWAN

ONU WAN connection is configured by private OMCI between OLT and ONU. When the connected ONU supports this function, the option "WAN" can be shown on this page.

Figure 4.1-11: Configure WAN

4.1.1.1.10 DHCP Server

ONU ConfigurationONU AuthListONU ListConfigDHCP Server

ONU LAN and DHCP server are configured by private OMCI between OLT and ONU. When the connected ONU supports this function, the option "DHCP Server" can be shown on this page.

Figure 4.1-12: ONU DHCP Server

4.1.1.1.11 Bind Mode

ONU ConfigurationONU AuthListONU ListConfigBIND Mode

ONU LAN bind mode is configured by private OMCI between OLT and ONU. When the connected ONU supports this function, the option "Bind Mode" can be shown on this page.

Figure 4.1-13: LAN Bind Mode Configuration

4.1.1.1.12 WIFI

ONU ConfigurationONU AuthListONU ListConfigWIFI

ONU WIFI is configured by private OMCI between OLT and ONU. When the connected ONU supports this function, the option "WIFI" can be shown on this page.

Figure 4.1-14: WIFI Configuration

4.1.1.1.13 VOIP

ONU ConfigurationONU AuthListONU ListConfigVOIP

This page shows WAN information of VOIP service, including IP address and VLAN. You can also operate VOIP module on this page. When the connected ONU supports VOIP, the option "VOIP" can be shown on this page.

Figure 4.1-15: Voice Wan Information

4.1.1.1.14 SIP

ONU ConfigurationONU AuthListONU ListConfigSIP

ONU VoIP SIP parameter can be configured on this page, including SIP server, proxy server, digit map and so on. When the connected ONU supports VOIP, the option "SIP" can be shown on this page.

Figure 4.1-16: SIP Parameter

4.1.1.1.15 POTS

ONU ConfigurationONU AuthListONU ListConfigPOTS

ONU VoIP POTS account, password and other VOIP parameters of POTS can be configured on this page; the length of SIP account can't be more than 16 bits. When the connected ONU supports VOIP, the option "POTS" can be shown on this page.

Figure 4.1-17: POTS Configuration

4.1.1.1.16 Misc

ONU ConfigurationONU AuthListONU ListConfigMisc

Misc includes other features of ONU which are configured by private OMCI.

Figure 4.1-18: Misc Configuration

4.1.1.2 Deactivate

ONU ConfigurationONU AuthListONU ListDeactivate (Activate)

Deactivate ONU which you selected, the ONU will be disabled and the registration failed. Activate selected ONU, this ONU will register successfully.

Figure 4.1-19: Deactivate/Activate ONU

4.1.1.3 Delete

ONU ConfigurationONU AuthListONU ListDelete

Delete ONU which you selected, the ONU will be deleted and the registration failed. All the configurations related this ONU will be deleted as well.

Figure 4.1-20: Delete ONU

4.1.1.4 Modify

ONU ConfigurationONU AuthListONU ListModify

This is used to modify ONU authentication mode.

Figure 4.1-21: Modify ONU Authentication mode

4.1.1.5 Optical Info

ONU ConfigurationONU AuthListONU ListOptical Info

Check the Optical Information of ONU PON module which you selected.

Figure 4.1-22: Optical info of ONU

4.1.1.6 Detail Info

ONU ConfigurationONU AuthListONU ListDetail Info

Check the Detail Info of ONU which you selected.

Figure 4.1-23: Detail info of ONU

4.1.1.7 Reboot

ONU ConfigurationONU AuthListONU ListReboot

Reboot ONU which you selected.

Figure 4.1-24: Reboot ONU

4.1.2 ONU Status

ONU ConfigurationONU AuthListONU Status

This pages shows the ONU information of the activity. User can check "Last Register Time”, "Last Deregister Reason" and "Active Time" of each ONU.

Figure 4.1-25: ONU Status

4.1.3 ONU Optical Info

ONU ConfigurationONU AuthListONU Optical Info

This page displays ONU Rx and Tx power. A batch of ONU optical power information can be shown in a list. Clearly to check the register power when register issue happens.

Figure 4.1-26: ONU Optical Info

4.1.4 ONU Version Information

ONU ConfigurationONU AuthListONU Version Information

This page allows you to view the optical power information of all online ONU.

Figure 4.1-27: ONU Version Information

4.1.5 ONU Manual Add

ONU ConfigurationONU AuthListONU Manual Add

You can manually add ONU to a selected PON port. ONU will appear in the ONU list after you added.

Figure 4.1-28: Add ONU Manually

4.1.6 ONU Allowist

ONU ConfigurationONU AuthListONU Allowist

You can set up Allowist on this page.

Whitelist can limit illegal ONU to register. Only the GPON SN in the Allowist can register, but only effective for the ONU which has not been added to OLT.

Figure 4.1-29: ONU Allowist

4.1.7 ONU Statistics

ONU ConfigurationONU AuthListONU Statistics

This page displays the information of package count about ONU ports.

Figure 4.1-30: ONU Statistics

4.2 ONU AutoFind

4.2.1 Automatic Discovery

ONU ConfigurationONU AutoFind Automatic Discovery

After selecting PON port number, all ONUs which are authenticated failed or not authenticated will be displayed in this interface. You can check the serial number of ONUs.

Figure4.2-1: Automatic Discovery

Figure 4.2-2: Detail info

4.2.2 Aging Time

ONU ConfigurationONU AutoFind Aging Time

Through this page, you can modify the aging time after discovering ONU through the PON port.

Figure4.2-3: Aging Time

4.3 ONU AutoLearn

4.3.1 ONU AutoLearn

ConfigurationAutoLearnONU AutoLearn

ONU can be authenticated automatically after enabling PON port automatic learning.

Figure 4.3-1: Automatic learn

4.3.2 ONU AutoBind

ConfigurationAutoLearnONU AutoBind

Input the Equipment ID and bind the profile you need

Note: you must create profile first.

Figure 4.3-2: Bind profile

4.3.3 ONU AutoDelete

ConfigurationAutoLearnONU AutoDelete

After this function is enabled, ONU registrations that are offline but remain offline for a certain period of time will be deleted.

Figure 4.3-3: ONU AutoDelete

4.3.4 ONU Scheduled Reboot

ConfigurationAutoLearnONU Scheduled Reboot

Through this page, you can set the scheduled restart function of ONU, it can specify one or more ONU to restart at a certain time.

Figure 4.3-4: ONU Scheduled Reboot

4.4 ONU Upgrade

ONU firmware can be upgraded by OLT. OLT supports manual upgrade and automatic upgrade.

4.4.1 UpLoad Image

ConfigurationONU UpgradeONU Image

Upload ONU firmware image which you need, the image will upload to OLT’s RAM.

Figure 4.4-1: Upload image

4.4.2 Manual Upgrade

ConfigurationONU UpgradeManual Upgrade

Select the ONU image and the ONU that need upgrade, click commit button to start upgrading. You can upgrade the ONU under one PON port everytime.

Figure 4.4-2: Manual Upgrade

4.4.3 Upgrade Status

ConfigurationONU UpgradeUpgrade Status

When ONU is upgrading, the upgrading status will be shown on this page.

Figure 4.4-3: ONU Upgrade Status

4.4.4 Auto Upgrade

ConfigurationONU UpgradeAuto Upgrade

After uploaded the ONU firmware image, configured automatic upgrade conditions, once the ONU which has the same equipment ID and different software version come online, they will be upgraded automatically.

Each ONU has its own equipment ID, which you can check in ONU detail info. Software version is the firmware image version which has uploaded to the OLT.

Figure 4.4-4: Auto Upgrade

4.4.5 Auto Upgrade Status

ConfigurationONU UpgradeAuto Upgrade Status

When ONU is auto upgrading, the upgrading status will be shown on this page.

Figure 4.4-5: Auto Upgrade Status

4.5 Rogue ONU

ONU ConfigurationRogue ONU

After enabled rogue ONU detect, if there is a rogue ONU trying to register, it will appear in the list.

Figure 4.5-1: Rogue ONU detect

4.6 ONU Common Service

ONU ConfigurationONU Common Service

You have more flexibility to create TCONT ID, Gemport, Service, Service port and Port Vlan for the specified ONU you select.

Figure 4.6-1: Create TCONT ID

Figure 4.6-2: Create Gemport

Figure 4.6-3: Create Service

Figure 4.6-4: Create Service port

Figure 4.6-5: Create Port Vlan

Chapter 5  Profile Configuration

This chapter is about the ONU profile configuration. It is designed for batch ONU management by OLT.

5.1 ONU Profile

The ONU profile is used for ONU authorization, and each ONU must specify only one ONU profile when authorization. The ONU profile specifies the capability of this ONU.

5.1.1 Information

Profile Configuration ONU profileInformation

The table displays ONU profile list. You can also do some operations, such as delete and check details info.

Figure 5.1-1: ONU profile list

5.1.2 Add profile

Create a new ONU profile what you need. Generally, ONU has two different modes.

SFU mode (only using bridge mode):

Usually, only need to set correct eth port and POTS port number of ONU, others can be kept default.

Figure 5.1-2: Add SFU profile

HGU mode (with the routing wan connection mode):

For HGU mode, need to set correct eth port and POTS port number and set veip to be 1, keep others default.

Figure 5.1-3: Add HGU profile

5.2 DBA Profile

DBA is a bandwidth allocation strategy that changes uplink bandwidth assigned to each T-CONT in real time according to the instant service status of each ONU. There are five BW types supported and make sure that fixed <= assured <= max.

5.2.1 DBA profiles

Profile ConfigurationDBA Profile DBA Profiles

The table displays DBA profile list. You can also do some operations, such as delete and modify.Default1 Profile defines bandwidth allocation for each of the three modes.

Figure 5.2-1: DBA profile list

5.2.2 Add profile

Profile ConfigurationDBA Profile  Add profile

There are five types of DBA profile. In general, we use type3.

Figure 5.2-2: Add a DBA profile

5.3 Traffic Profile

Traffic profile is used by gemport to specify the upstream/downstream bandwidth.

5.3.1 Traffic profiles

Profile ConfigurationTraffic Profile  Traffic Profiles

The table displays Traffic profile list. You can also do some operation, such as delete and modify.

Figure 5.3-1: Traffic Profile list

5.3.2 Add profile

Profile ConfigurationTraffic Profile  Add Profile

Configure gemport to specify the upstream/downstream bandwidth.

SIR: Committed Information Rate

CBS: Committed Burst Size

Figure 5.3-2: Add a traffic Profile

5.4 Line Profile

Line profile is used to configure the ANI side services of ONU such as t-cont, gem-port, service-port, and so on.

5.4.1 Line profile

Profile ConfigurationLine Profile  Line Profile

The table displays Line profile list. You can also do some operations, such as delete and modify.

Figure 5.4-1: Line Profile list

5.4.2 Add profile

Profile ConfigurationLine profileAdd profile

Create a new line profile.

Figure 5.4-2: Add Line Profile

Modify the line profile parameters.

Figure 5.4-3: Modify Line Profile

5.4.2.1 Tcont

Add tcont ID and bind DBA profile.

Figure 5.4-4: Add Tcont

5.4.2.2 Gemport

Add gemport ID and bind tcont ID.

Figure 5.4-5: Add Gemport

5.4.2.3 Service

Add service, set the VLAN mode and VLAN ID and bind one gemport ID.

Figure 5.4-6: Add Service

5.4.2.4 Service Port

Create a service port, set the user VLAN and translate VLAN and bind one gemport ID. If don’t need VLAN translation, just set translate VLAN the same as user VLAN.

Figure 5.4-7: Add Service Port

5.4.2.5 Multicast Vlan

Set the Multicast VLAN of ONU.

Figure 5.4-8: Configure Multicast VLAN

5.5 Service Profile

Service profile is used to configure the UNI side services of onu, such as Ethernet port, wifi, veip, and so on.

5.5.1 Service profile

Profile ConfigurationService Profile  Service Profile

The table displays service profile list. You can also do some operations, such as delete and modify.

Figure 5.5-1: Service Profile List

5.5.2 Add profile

Profile ConfigurationService Profile Add Profile

Add a new service profile.

Figure 5.5-2: Add Service profile

Figure 5.5-3: Modify Service Profile

5.5.2.1 PortVlan

Set the VLAN mode of the ONU’s port. For HGU, need to configure veip 1 transparent; for SFU, configure Ethernet port directly.

Figure 5.5-4: Port VLAN mode

5.5.2.2 Multicast Vlan Strip

Set the multicast VLAN mode of ONU’s port.

Figure 5.5-5: Port Multicast VLAN Mode

5.5.2.3 Port

Configure the status and rate of the onu port

Figure 5.5-6: Port status

5.5.2.4 Iphost Config

Add Iphost for ONU wan connection. IPhost is used for ONU management.

Figure 5.5-7: Add IPhost

5.6 Alarm Profile

Alarm profile is used to configure the parameters of ONU alarm.

5.6.1 Profile Infomation

Profile ConfigurationAlarm Profile profile infomation

The table displays alarm profile list.

Figure 5.6-1: Alarm Profile List

5.6.2 Add Profile

Profile ConfigurationAlarm Profile Add profile

Add new alarm profile, set the threshold of alarm generation.

Figure 5.6-2: Add Alarm Profile

5.7 Pri Profile

Pri Profile is the profile which the parameters are configured by private OMCI, including WAN, SIP, WIFI, CATV, DHCP Server, and so on.

5.7.1 Pri Profile

Profile ConfigurationPri Profile

The table displays private profile list. You can also do some operations, such as delete and modify.

Figure 5.7-1: Pri Profile

5.7.2 Add Profile

Profile ConfigurationPri Profile Add profile

Figure 5.7-2: Add Private Profile

5.8 IGMP Profile

IGMP profile is used to configure the parameters of ONU multicast .

5.8.1 IGMP Profile

Profile ConfigurationIGMP Profile IGMP Profile

The table displays IGMP profile list.

Figure 5.8-1: IGMP Profile List

5.8.2 Add Profile

Profile ConfigurationIGMP Profile Add profile

Add new IGMP profile, set the threshold of alarm generation.

Figure 5.8-2: Add IGMP Profile

Figure 5.8-3: Modify IGMP Profile

5.8.2.1 Config

Configure multicast related data.

Figure 5.8-4: Config

5.9 Format Profile

Format profile is used to configure the parameters of ONU DHCP related information .

5.9.1 Format Profile

Profile ConfigurationFormat Profile Format Profile

The table displays Format profile list.

Figure 5.9-1: Format Profile List

5.9.2 Add Profile

Profile ConfigurationFormat Profile Add profile

Add new Format profile.

Figure 5.9-2: Add Format Profile

Figure 5.9-3: Modify Format Profile

5.9.2.1 Config

Configure Format related data.

Figure 5.9-4: Config

5.10 Bind Profile

After profile is configured, it is necessary to bind it to ONU.

Profile ConfigurationBind Profile

Figure 5.10-1: Bind profile

Figure 5.10-2: Select Profile

Chapter 6  System Configuration

This chapter is about the global management of OLT.

6.1 System Log

6.1.1 System Log

System ConfigurationSystem Log

This page displays OLT system alarms and events.

Figure 6.1-1: System Log

6.1.2 Alarm

System Configuration System Log Alarm

It contains all the alarms of OLT. User can choose the different alarms to "Print", "Record", "Trap" and "Remote".

Figure 6.1-2: Alarm

| options | Illustration |
| --- | --- |
| Print | Alarm and event show in console and telnet, but not show in syslog, EMS and remote log server. |
| Record | Alarm and event show in syslog, but not show in console, telnet, EMS and remote log server. |
| Trap | Alarm and event show in EMS, but not show in console, telnet, syslog and remote log server. |
| Remote | Alarm and event show in remote log server, but not show in console, telnet, syslog and EMS. |

6.1.3 Threshold Alarm

System Configuration System Log Threshold Alarm

This page is used to configure OLT temperature threshold, CPU-usage threshold and memory- usage threshold, PON optical threshold.

Figure 6.1-3: Threshold Alarm

6.1.4 Syslog Server

System ConfigurationSystem Log Syslog Server

This page is used to configure remote IPv4 server of OLT system log.

Figure 6.1-4: Syslog Server

6.1.5 Syslog Server IPv6

System ConfigurationSystem Log Syslog Server IPv6

This page is used to configure remote IPv6 server of OLT system log.

Figure 6.1-5: Syslog Server IPv6

6.2 Device Management

6.2.1 Firmware Upgrade

System ConfigurationDevice Management Firmware Upgrade

You can upgrade the OLT firmware on this page. OLT will reboot automatically with the new firmware after upgraded.

Figure 6.2-1: Firmware Upgrade

6.2.2 Device Reboot

System ConfigurationDevice Management Device Reboot

You can reboot the entire system on this page. Please do save the configuration before reboot.

Figure 6.2-2: Device Reboot

6.2.3 Config File

System ConfigurationDevice Management Config File

You can backup configuration, restore configuration, restore factory defaults and save configuration on this page.

Figure 6.2-3: File Configuration

6.2.4 Advance Config File

System ConfigurationDevice Management Advance Config File

You can choose between periodic auto-save configuration and periodic auto-backup configuration, There are three modes: Timeout, Fix-Time and Week-Day.

Figure 6.2-4: Auto Save

6.3 User Management

6.3.1 User manage

System ConfigurationUser managementUser manage

Two types of user have been defined, Normal and Admin. There are limitations to normal user, and Admin user has no limits to full function of OLT. The default account member is Admin level.

Figure6.3-1: User Manage

6.3.2 Role manage

System ConfigurationUser managementRole manage

You can add roles and assign different permissions to different roles.

Figure6.3-2: Role Manage

6.4 SNMP

6.4.1 SNMP V1/V2

System Configuration  SNMP SNMP V1/V2

This page is used to configure SNMP parameters of version 1 and version 2 for OLT management.

Figure6.4-1: SNMP V1/V2

6.4.2 SNMP V3

System Configuration  SNMP SNMP V3

This page is used to configure SNMP parameters of version 3 for OLT management.

Figure6.4-2: SNMP V3

6.4.3 SMNP V3 Trap

System Configuration  SNMP SNMP V3 Trap

Configure the target host IP address of trap messages.

Figure 6.4-3: SNMP V3 Trap

6.4.4 Remote Server

System Configuration  SNMP Remote Server

Configure the IP address of your SNMP network management server.

Figure 6.4-4: Remote Server

6.5 AUX IP

6.5.1 AUX IP

System Configuration  AUX IP AUX IP

AUX port is out band management port. The IP address of aux port is out band management IP. Default IPv4 address is 192.168.8.200.

Figure 6.5-1: AUX IP

6.5.2 AUX IPv6

System Configuration  AUX IP  AUX IPv6

AUX port is out band management port. The IP address of aux port is out band management IP. By default, there is a link local address.

Figure 6.5-2: AUX IPv6

6. 6 DNS

DNS is used for domain name resolution. When OLT need to visit a site or a destination by domain, take NTP server for example, DNS is required.

6.6.1 IPv4 DNS

System Configuration  DNS  IPv4 DNS

This page is used to configure IPv4 DNS.

Figure 6.6-1: IPv4 DNS

6.6.2 IPv6 DNS

System Configuration  DNS  IPv6 DNS

This page is used to configure IPv6 DNS.

Figure 6.6-2: IPv6 DNS

6.7 System Time

6.7.1 RTC

System Configuration  System TimeRTC

This page is used to set OLT system time. RTC stands for Real-Time Clock, it provides clock signal to the system. There is no battery inside OLT, so the time will not be saved after powered off.

Figure 6.7-1: RTC Setting

6.7.2 NTP

System Configuration  System TimeNTP

This page is used to configure NTP server. OLT will synchronize time with the NTP server at a given time.

Figure 6.7-2: NTP Configuration

6.8 FAN

System Configuration  FAN

The fans can be turned on and turned off manually; and also can be turned on and off automatically according to the temperature of OLT main chip.

This configuration will not be saved after reboot.

Figure 6.8-1: FAN Configuration

6.9 Mirror

System Configuration  Mirror

Port mirror is usually used for troubleshooting. Each monitor session can be set with one destination port and up to 5 source ports.

Figure 6.9-1: Mirror Configuration

6.10 Login Management

6.10.1 Login Access List

System Configuration  Login Management Login Access List

This page is used to configure access rights for management. You can configure access rights for telnet, web, SNMP, SSH according to source IP address.

Figure 6.10-1:  Login Access List Configuration

6.10.2 Service Port

System Configuration  Login Management Service Port

This user interface allows you to modify the default remote service port.

Figure 6.10-2:  Service Port Configuration

6.10.3 Login Timeout

System Configuration  Login Management Login Timeout

This page is used to set web timeout.

Figure 6.10-3:  Login Timeout Configuration

6.10.4 Telnet Management

System Configuration  Login Management Telnet Management

Here you can see the user who telnet into OLT.

Figure 6.10-4:  Telnet Management

6.11 SSH

SSH (Secure Shell) is a reliable protocol that provides security for remote login sessions and other network services. The SSH protocol can effectively prevent information leakage during remote management.

6.11.1 SSH State

System Configuration  SSH SSH State

This page displays current connections that have established by SSH protocol.

Figure 6.12-1: SSH State

6.11.2 SSH Enable

System Configuration  SSH SSH Enable

This page is used to configure SSH protocol related parameters.

Figure 6.12-1: SSH Global Configuration

6.12 Tacacs+

Tacacs+ is a protocol that provides access control for routers, network access servers, and other interconnected computing devices through one or more centralized servers. Tacacs+ provides independent authentication, authorization, and billing services.This interface allows you to configure the Tacacs+ server IP address and other specific parameters.

Figure 6.14-1: Tacacs+ Configuration

6.13 Radius

Radius is a protocol for authentication, authorization, and accounting information.The Radius server is responsible for receiving the user's connection request, authenticating the user, and then returning all the necessary configuration information to the client to send the service to the user.This interface allows you to configure the Radius server IP address and other  parameters.

Figure 6.15-1: Radius Configuration

6.14 Dot1x

802.1x is a Client/ server-based access control and authentication protocol. It can restrict unauthorized users/devices from accessing a LAN/WLAN through an access port. After the authentication, normal data can pass through the Ethernet port.

6.14.1 Dot1x Information

This interface will display dot1x entry information when an 802.1x user passes authentication on the server, the server sends the authorization information to the device.

Figure 6.14-1:Dot1x Information

6.14.2 Dot1x Global Enable

You can enable Dot1x on this interface.

Figure 6.14-2:Dot1x Global Enable

6.14.3 Port Configuration

You can configure detailed Dot1x parameters for a specified port on this interface.

Figure 6.14-3:Port Configuration

6.15 Network Diagnose

6.15.1 Ping Diagnose

System Configuration  Diagnose Ping Diagnose

This interface is used to diagnose network connectivity.

Figure 6.15-1: Ping Diagnose Configuration

6.15.2 Tracert Diagnose

System Configuration  Diagnose Tracert Diagnose

This interface is used to track and diagnose routing and forwarding.

Figure 6.15-2: Tracert Diagnose Configuration

6.15.3 Netstat Diagnose

System Configuration  Diagnose Netstat Diagnose

This is a network test.

Figure 6.15-3: Netstat Diagnose Configuration

6.15.4 Tcpdump Diagnose

System Configuration  Diagnose Tcpdump Diagnose

This is the PCP link test.

Figure 6.15-4: Tcpdump Diagnose Configuration

6.15.5 Iperf Diagnose

System Configuration  Diagnose Iperf Diagnose

This is Iperf-related testing.

Figure 6.15-5: Iperf Diagnose Configuration

Thank you!